Lab 4

# Getting started with Open Source Cloud Server Upload any Sensor Data to cloud Server ThingSpeak Complete Tutorial Read, Write with Example

## ThingSpeak:

There are two ways one is hard way without library and another with library

### Video Tutorial can be Found Below Link:

- Hard way

# https://www.youtube.com/watch?v=jYeUS7JxmA8

- Easy way:

https://www.youtube.com/watch?v=HM7_Urqoric

# Step 1: Go thingspeak website and Create your Account

After creating your Account we ned to copy Read and Write API Keys shown in image below

# Step 2: Copy Credentials all all API keys

# We are Going to Learn   Both ways

- Upload the Data with 3 lines of code using Library that i have developed

- Upload using URLlib and request Module

# Method 1: Harder way

Write Data

import urllib.request

import requests

import threading

import json

import random

def thingspeak_post():

threading.Timer(15,thingspeak_post).start()

val=random.randint(1,30)

URl='https://api.thingspeak.com/update?api_key='

KEY=' -------WRITE KEY----------'

HEADER='&field1={}&field2={}'.format(val,val)

NEW_URL = URl+KEY+HEADER

print(NEW_URL)

data=urllib.request.urlopen(NEW_URL)

print(data)

if __name__ == '__main__':

thingspeak_post()

# Method 2 Easy way with Three Line of Code

## important Note:

Assume your name of your python file is main and the code that i am about to show you is the library developed to do jon in 3 lines of code. Paster the Entire 500 Line of code in your File called main and all way down start writing your code

or

copy the library ie python file in your working Directory and you can simple say from nameofpythonfile import *

# Assume your Work Directory is

Foldername ---- IoTclass

> main.py

> masterclass.py

in your main Python File you will say from masterclass import *

if that dosent work for some reason you can also copy entire 500 Line of library code in main.py and all the way down start writing your code

## Code

from masterclass import *

temperature = 22

humidity = 11

w_key = 'AZRMN9ZP5FKLXYLR'

r_key = '17FSCQ4FTX6V4VWM'

channel_id = 83234

ob = Thingspeak(write_api_key=w_key, read_api_key=r_key, channel_id=channel_id)

ob.post_cloud(value1=temperature,value2=humidity)

# Sample Code using Master class.

### Use this Template for Coding !

try:

import numpy as np

from masterclass import *

except:

print("Library not Found ! ")

x  = []

y1 = []

y2 = []

def sensor_random_value():

"""

Write you Raspberry pi Sensor Code Inside this

:return: Comma Seperated  Values

"""

x = np.random.randint(0, 20, None)              # create random value which resenble to sensor data Live

y = np.random.randint(0, 20, None)              # create a another random variable to store Data Sensor 2

my_sensor = "{},{}".format(x, y)                # create a Fake Data which represent the hardware data

return my_sensor

def mymain():

x,y = sensor_random_value().split(',')

w_key = 'AZRMN9ZP5FKLXYLR'

r_key = '17FSCQ4FTX6V4VWM'

channel_id = 83234

ob = Thingspeak(write_api_key=w_key, read_api_key=r_key, channel_id=channel_id)

ob.post_cloud(value1=x,value2=y)

if __name__ == "__main__":

mymain()

Task 1: Upload DHT-11 Sensor Data to Cloud Server !

try:

import Adafruit_DHT

from masterclass import *

except:

print("Library not Found !")

def sensor_value():

# Example using a Raspberry Pi with DHT sensor

# connected to GPIO23.

pin = 4

sensor = Adafruit_DHT.DHT22

humidity, temperature = Adafruit_DHT.read_retry(sensor, pin)

if humidity is not None and temperature is not None:

print('Temp={0:0.1f}*C  Humidity={1:0.1f}%'.format(temperature, humidity))

data = '{},{}'.format(temperature,humidity)

return data

else:

print('Failed to get reading. Try again!')

def my_main():

x,y = sensor_value().split(',')

w_key = 'AZRMN9ZP5FKLXYLR'

r_key = '17FSCQ4FTX6V4VWM'

channel_id = 83234

ob = Thingspeak(write_api_key=w_key, read_api_key=r_key, channel_id=channel_id)

ob.post_cloud(value1=x,value2=y)

if __name__ == "__main__":

my_main()

# Reading Data From Cloud

# print(ob.read_cloud(result=3))

## Task: Try to interface Distance Sensor and Upload the Data to Cloud Server.

# For Those who want to see my Library Code

# https://github.com/soumilshah1995/IoTMaster

"""" Author : Soumil shah Email : soushah@my.bridgeport.edu Version 1.0.1 update: Changes made to class added private var and url and read url cannot be changed ! """ try: # import the important library from urllib import request from urllib.request import urlopen import threading # import threadding import json # import json import random # import random import requests # import requests for web API import ssl import geocoder # for Locations import datetime # for date time from twilio.rest import Client # for Sms from serial import Serial # for arduino import numpy as np from Adafruit_IO import Client # for Adafruit IO import os from os import system # For Text to speech import paho.mqtt.publish as publish import sqlite3 # for Database import paho.mqtt.publish as publish import urllib # for web Api except: print("No Library Found") class Thingspeak(object): # define a class called Thingspeak def __init__(self, write_api_key = None, read_api_key=None, channel_id=0): """ :param write_key: takes a string of write api key :param timer: can take integer values """ # self.url = 'https://api.thingspeak.com/update?api_key=' # self.read_url = 'https://api.thingspeak.com/channels/{}/feeds.json?api_key='.format(channel_id) self.write_key = write_api_key self.channel_id = channel_id self.read_api_key = read_api_key # Private Var cannot change self.__url = 'http://api.thingspeak.com/update?api_key' self.__read_url = 'https://api.thingspeak.com/channels/{}/feeds.json?api_key='.format(channel_id) self.feild1 = [] self.feild2 = [] def post_cloud(self, value1, value2): try: """ :param value1: can be interger or float :param value2: can be interger or float :return: updated to cloud storage """ URL = self.__url KEY = self.write_key HEADER = '&field1={}&field2={}'.format(str(value1), str(value2)) NEW_URL = str(URL) + "=" + str(KEY) + str(HEADER) print(NEW_URL) context = ssl._create_unverified_context() data = request.urlopen(NEW_URL,context=context) print(data) except: print('could not post to the cloud server ') def read_cloud(self, result=1): try: """ :param result: how many data you want to fetch accept interger :return: Two List which contains Sensor data """ URL_R = self.__read_url read_key = self.read_api_key header_r = '&results={}'.format(result) new_read_url = URL_R + read_key + header_r data = requests.get(new_read_url).json() field1 = data['feeds'] for x in field1: self.feild1.append(x['field1']) self.feild2.append(x['field2']) return self.feild1, self.feild2 except: print('read_cloud failed !!!! ') class IfTTT(object): def __init__(self, eventname='', key=''): self.eventname = eventname self.Key = key self.__Url = 'https://maker.ifttt.com/trigger/{}/with/key/'.format(self.eventname) self.New_Url = self.__Url + self.Key print(self.New_Url) def iftt_post(self, data1=10, data2=11): try: """ :param data1: pass your sensor value only integer :param data2: pass your data as interger only :return: True if it was successful """ URl = self.New_Url Key = self.Key payload = {'value1': data1, 'value2': data2} requests.post(self.New_Url, data=payload) print("Done posted on IFTTT") return True except: print('failed to post to cloud sever ! ') class Location(object): def __init__(self): pass def get_locations(self): """ :return: Lat and Long """ try: g = geocoder.ip('me') my_string=g.latlng longitude=my_string[0] latitude=my_string[1] return longitude,latitude except: print('Error make sure you have Geo-Coder Installed ') class DateandTime(object): def __init__(self): pass @ staticmethod def get_time_date(): try: """ :return: date and time """ my = datetime.datetime.now() data_time = '{}:{}:{}'.format(my.hour,my.minute,my.second) data_date = '{}/{}/{}'.format(my.day,my.month,my.year) return data_date,data_time except: print('could now get date and time ') def convert_timestamp(self,timestamp): timestamp = 1554506464 dt_object = datetime.fromtimestamp(timestamp) return dt_object class Weather_details(object): def __init__(self,key='', city=''): self.city = city self.key = key def get_weather_data(self): try: city = self.city key = self.key URL='http://api.openweathermap.org/data/2.5/weather?appid={}&q={}'.format(key,city) print(URL) data = requests.get(URL).json() long = data['coord']['lon'] lat = data['coord']['lat'] humidity = data['main']['humidity'] wind_speed = data['wind']['speed'] wind_degree = data['wind']['deg'] sunrise = data['sys']['sunrise'] sunset = data['sys']['sunset'] m1 = data['weather'][0]['description'] m2 = data['weather'][0]['main'] body = '{} {}'.format(m1, m2) return long, lat, humidity, wind_speed, wind_degree, sunrise, sunset,body except: print('Error occured ') class Arduino(object): def __init__(self,comport='/dev/cu.usbmodem14101', baudrate=9600): self.comport = comport self.baudrate = baudrate def read_data(self): try: """ :return: data after reading from serial object """ arduino = Serial(self.comport, self.baudrate,timeout=1) data = arduino.readline().decode('ascii') return data except: print("please check your com port and baud rate ") def write_data(self,data='1'): try: """ :param data: pass string 1 or 0 or any character and write if statment in Arduino to compare and trigger events :return: NONE """ arduinodata = Serial(self.comport, self.baudrate,timeout=1) data_send = data.encode('utf-8') arduinodata.write(data_send) except: print("cannot write data please check com port and baud rate ") class Adafruit_cloud(): def __init__(self, username='', Aio_key=''): self.username = username self.Aio_key = Aio_key self.aio = Client(self.username,self.Aio_key) self.feed_name = [] def adafruit_send(self,feed_name='',data=''): try: sensor = self.aio.feeds(feed_name) self.aio.send_data(sensor.key,data) print('Data was uploaded ') except: print("cannot send Data !") def adafruit_get(self, feedname=''): try: data = self.aio.receive(feedname) return data.value except: print('cannot get data ! ') def adafruit_feed_list(self): try: feeds = self.aio.feeds() for f in feeds: print(f) self.feed_name.append(f.name) return self.feed_name except: print("cannot get feed list ! ") class TextSpeech_Mac(object): def __init__(self): """ This script works for mac only """ pass def speak(self,text): try: """ :param text: Takes String as Input :return: Return None """ system("say {}".format(text)) except: print("This works for mac only ! ") class Mqtt(object): def __init__(self, data): self.data= data self.hostname="test.mosquitto.org" self.topic = '' def post(self): """ POst teh :return: """ publish.single(self.topic, "{}".format(self.data), hostname=self.hostname) class YoutubeSub(object): def __init__(self,name ='',google_key=''): """ :param name: Name of Youtube Person , Google Cloud API key :param google_key: """ self.name = name self.google_key = google_key self.context = ssl._create_unverified_context() def get_subscriber(self): """ :return: Youtube Sub Count """ self.url = 'https://www.googleapis.com/youtube/v3/channels?part=statistics&forUsername=' \ + self.name+'&key='+self.google_key data = urllib.request.urlopen(self.url, context=self.context).read() subs = json.loads(data)["items"][0]["statistics"]["subscriberCount"] subs = int(subs) return subs class Text_MP3_converter(object): def __init__(self): self.__url = 'https://text-to-speech-demo.ng.bluemix.net/api/v1/synthesize?t' self.__header ={ 'Accept-Encoding': 'gzip, deflate, sdch', 'Accept-Language': 'en-US,en;q=0.8', 'User-Agent': 'Mozilla/5.0 (Macintosh; Intel Mac OS X 10_10_1) AppleWebKit/537.36 (KHTML, like Gecko) Chrome/39.0.2171.95 Safari/537.36', 'Accept': 'text/html,application/xhtml+xml,application/xml;q=0.9,image/webp,*/*;q=0.8', 'Referer': 'http://www.wikipedia.org/', 'Connection': 'keep-alive'} self.__params = { 'text': 'hello everyone i am going to teach you python', 'voice': 'en-US_AllisonV2Voice', 'download': True, 'accept': 'audio/mp3'} def text_audio(self,name ='test', text='hello world'): """ :param name_file: :param text: :return: saves MP3 File on your computer """ try: response = requests.get(self.__url, headers=self.__header,params=self.__params) with open("{}.mp3".format(name),'wb') as f: f.write(response.content) print("File has been downloaded on your computer with name {}".format(name)) except: print('Error cannot convert File') class Spotifypy(object): def __init__(self,client_id='', client_secret='',oauth_token=''): self.client_id = client_id self.client_secret = client_secret self.oauth_token = oauth_token self.__header = {'Authorization': self.oauth_token, 'Accept':'application/json', 'Content-Type':'application/json'} def play_song(self): self.__url_play = 'https://api.spotify.com/v1/me/player/play' response = requests.put(self.__url_play, headers=self.__header) print(response) def pause_song(self): self.__url_pause = 'https://api.spotify.com/v1/me/player/pause' response = requests.put(self.__url_pause, headers=self.__header) print(response) def get_recommendation(self): self.__url_recommendation = 'https://api.spotify.com/v1/recommendations/available-genre-seeds' response = requests.get(self.__url_recommendation, headers=self.__header) # print(response.json()) return response.json() def get_device(self): self.__url_device = 'https://api.spotify.com/v1/me/player/devices' response = requests.get(self.__url_recommendation, headers=self.__header) return response.json() class Converter(object): def __init__(self): self.__url = "https://www.binaryhexconverter.com/hesapla.php" self.__params = { 'fonksiyon': 'dec2hex', 'deger': 12, 'pad': False, 'v': 2 } def bin2hex(self,binnum=1): self.__params['deger'] = binnum self.__params['fonksiyon'] = 'bin2hex' response = requests.get(self.__url,params=self.__params) return response.text def bin2dec(self, binnum=1): self.__params['deger'] = binnum self.__params['fonksiyon'] = 'bin2dec' response = requests.get(self.__url, params=self.__params) return response.text def bin2asc(self,binnum=1): self.__params['deger'] = binnum self.__params['fonksiyon'] = 'bin2asc' response = requests.get(self.__url, params=self.__params) return response.text def hex2bin(self, hexnum ='A'): self.__params['deger'] = hexnum self.__params['fonksiyon'] = 'hex2bin' response = requests.get(self.__url, params=self.__params) return response.text def hex2dec(self,hexnum ='A'): self.__params['deger'] = hexnum self.__params['fonksiyon'] = 'hex2dec' response = requests.get(self.__url, params=self.__params) return response.text def dec2hex(self, decnum='10'): self.__params['deger'] = decnum self.__params['fonksiyon'] = 'dec2hex' response = requests.get(self.__url, params=self.__params) return response.text def dec2bin(self, decnum='10'): self.__params['deger'] = decnum self.__params['fonksiyon'] = 'dec2bin' response = requests.get(self.__url, params=self.__params) return response.text def dec2oct(self, decnum='10'): self.__params['deger'] = decnum self.__params['fonksiyon'] = 'dec2oct' response = requests.get(self.__url, params=self.__params) return response.text

# Getting Started with Twilio and Raspberry pi for SMS Notification [Lab 4A]

## objective:

In this we are going to Learn how to use Twilio API with Python and Raspberry to Send SMS when Event is Occured. Event can be When Temperature Exceeds more than Specified Value it shall send a SMS to a contact numnber.

# Step 1: Create a Account on Twilio

Go to Twilio Website and Create a Accout.

https://www.twilio.com

# Step 2: Get Twilo Phone Number

After you have created a account. go ahead and get a Twilio Phone Number and also we would require Twilio AUTH KEY and SSID go ahead and get the Twilio Number.

## After you get Twilo Number you should see a screen like this

# Step 3: Download Python Library for Twilio

pip install twilio.rest if pip install didnt work for you go ahead to google and and read the doccumentaion on downloading TWILIO for Python 3 +

# Step 4 : Lets Code .........

### Import the library

from twilio.rest import Client

### Define a Function to send SMS

Make sure to use try and except Block to Avoid Errors Add you SSID and AUTH Token.

# def send_sms_alert():

# try:

# # Define your body

# my_body='Yo'

# # define client

# client = Client('SSID GOES HERE XXX ','AUTH TOKEN XX ')

# client.messages.create(to='+ TO XXXX ',

# from_= '+TWILIO NUMBER XXX',

# body=my_body)

# except:

# print('Cannot send Sms!')

### Putting all Together

# from twilio.rest import Client

# def send_sms_alert():

# try:

# # Define your body

# my_body='Yo'

# # define client

# client = Client('SSID GOES HERE XXX ','AUTH TOKEN XX ')

# client.messages.create(to='+ TO XXXX ',

# from_= '+TWILIO NUMBER XXX',

# body=my_body)

# except:

# print('Cannot send Sms!')

# send_sms_alert()

If you have Error make sure you have entered correct SSID and AUTH key. After that Make sure you have entered correct Phone Number Make sure to senter your number in to as we are suing Trial Account wont be able to send sms to other people as its a premuim Features.

## Congrulations You just sent a SMS using Python¶

# Practical Example

# Please follow complete Steps on how to install the Library on following link

Installation Steps: -

Python library to read the DHT series of humidity and temperature sensors on a Raspberry Pi or Beaglebone Black. Designed specifically to work with the Adafruit DHT series sensors ----> https://www.adafruit.com/products/385 Currently the library is tested with Python 2.6, 2.7, 3.3 and 3.4. It should work with Python greater than 3.4, too Dependencies

For all platforms (Raspberry Pi and Beaglebone Black) make sure your system is able to compile and download Python extensions with pip:

On Raspbian or Beaglebone Black's Debian/Ubuntu image you can ensure your system is ready by running one or two of the following sets of commands

Python 2: sudo apt-get update sudo apt-get install python-pip sudo python -m pip install --upgrade pip setuptools wheel

Python 3: sudo apt-get update sudo apt-get install python3-pip sudo python3 -m pip install --upgrade pip setuptools wheel

Install with pip Use pip to install from PyPI.

Python 2: sudo pip install Adafruit_DHT

Python 3: sudo pip3 install Adafruit_DHT

Compile and install from the repository First download the library source code from the GitHub releases page, unzipping the archive, and execute: Python 2: cd Adafruit_Python_DHT sudo python setup.py install

Python 3: cd Adafruit_Python_DHT sudo python3 setup.py install

You may also git clone the repository if you want to test an unreleased version: git clone https://github.com/adafruit/Adafruit_Python_DHT.git

import Adafruit_DHT

from twilio.rest import Client

pin = 4

sensor = Adafruit_DHT.DHT22

def main():

humidity, temperature = Adafruit_DHT.read_retry(sensor, pin)

if humidity is not None and temperature is not None:

print('Temp={0:0.1f}*C  Humidity={1:0.1f}%'.format(temperature, humidity))

if humidity >34:

print("Alert ")

send_sms_alert(' High Temperature Alert Warning ')

else:

print('Failed to get reading. Try again!')

def send_sms_alert(text='Python'):

try:

# Define your body

my_body = text

# define client

client = Client('SSID GOES HERE XXX ','AUTH TOKEN XX ')

client.messages.create(to='+ TO XXXX ',

from_= '+TWILIO NUMBER XXX',

body=my_body)

except:

print('Cannot send Sms!')

if __name__ == "__main__":

while True:

main()

Application

Here is an example where I used Twilio

https://www.youtube.com/watch?v=hT4zp-mGFkE

https://www.youtube.com/watch?v=lrSUts2hNlw

Getting started with IFTTT Lab 4B

# Objective:

our goal is to learn the Most popular Cloud services that is IFTTT. IFTTT stands for if this than that. it provides lot of services from Phone, SMS, Gmail , Drive and much more possibilities are endless. it also allows you to connect your IoT device get and notidfied when an event occurs through webhook.

### Step 1: Create a Account on IFTTT

once you create a accout you should see a screen something like this

### Step 2: Search for WebHook on Search Bar

### Step 3: Click on Documentation

### Step 4: Copy the Secret Key

### Step 5: create a New Applete

Choose receive web requests option form menu

Create Trigger

Note:- Use the IoT Master Library which was discussed above

Code:

#iftt_k = 'Q-XXXXXX-7Xo98chRB_'
#feed = 'sensor'

#ob1 = IfTTT(eventname=feed, key=iftt_k)
#ob1.iftt_post(data1=temperature,data2=humidity)